ММОСО'Н — модуль моделирования орбитальной станции О'Нилла

Раздел 34 / Section 34

34. ГОРОДА: РЕАЛЬНЫЕ И ВИРТУАЛЬНЫЕ

34. CITIES: REAL AND VIRTUAL

Редакция 8 · 31 октября 2026 г.  ·  документ 36 из 43

——————————————————————————————————

РУССКАЯ РЕДАКЦИЯ / РУССКИЙ ТЕКСТ

# 34. ГОРОДА: РЕАЛЬНЫЕ И ВИРТУАЛЬНЫЕ

Пункт 12.5 описывает урбанистику как деление обитаемой поверхности. Настоящий раздел отвечает на два вопроса: как должны быть устроены города станции, и как их проектировать, когда полномасштабный макет невозможен.

## 34.1. Плотность и площадь

Обитаемая площадь двух цилиндров составляет 2 513,2 км² при населении 1 000 000, то есть плотность 398 человек/км². Это в пять раз ниже плотности типичного земного города и в несколько раз выше сельской. При норме 120 м² застройки на человека города занимают 120 км², или 4,8 % обитаемой поверхности; открытые пространства при норме 25 м² на человека — 25 км².

Таблица 34.1 — Городские показатели станции

| Показатель | Величина |
| --- | --- |
| Обитаемая площадь | 2 513,2 км² |
| Плотность | 398 человек/км² |
| Застройка | 120 км² (4,8 %) |
| Открытые пространства | 25 км² |
| Городов по 50 000 человек | 20 |
| Площадь одного города | 6,0 км² |
| Радиус города | 1,38 км |

## 34.2. Реальный город: рекомендации

Двадцать городов по 50 000 человек, а не одно метро: каждый город должен быть самодостаточен по питанию, воде, энергии и аварийному секционированию.

Радиус 1,38 км при радиусе цилиндра 5 км: город занимает меньше половины ширины цилиндра, и пешая доступность сохраняется во всех направлениях.

Зонирование по климатическим поясам (п. 2.4): город в тропической зоне и город в холодной зоне — разные инженерные задачи, и их нельзя проектировать одним типовым решением.

Ориентация: продольная ось цилиндра — единственное направление, где горизонт не ограничен (п. 22.1). Улицы и виды должны быть ориентированы преимущественно вдоль оси.

Полоса тьмы: половина длины цилиндра лишена прямого света ОСБ; города должны проектироваться с учётом искусственного освещения и светового климата (п. 22.3).

Акустика: собственные частоты оболочки лежат ниже полосы слышимого звука (п. 7.2), и городской шум не находит отражения от стен — планировка должна быть открытой.

## 34.3. Виртуальный двойник в Minecraft

Полномасштабный макет станции невозможен: цилиндр длиной 49 км и диаметром 10 км не помещается ни в одном павильоне. Виртуальный двойник решает эту задачу, и Minecraft оказывается пригоден потому, что его блочная геометрия естественно соответствует ортогональной планировке цилиндра.

Таблица 34.2 — Виртуальный двойник: масштабы

| Масштаб | Размер, блоков | Поверхность | Пригодность |
| --- | --- | --- | --- |
| 1:1 | 49 000 × 10 000 | 490 млн | Не помещается по высоте |
| 1:100 | 490 × 100 | 49 000 | Рекомендуемый |
| 1:200 | 245 × 50 | 12 250 | Обзорный |
| 1:1000 | 49 × 10 | 490 | Слишком грубый |

Масштаб 1:1 технически достижим по длине и по диаметру в плане, но не по высоте: диаметр 10 000 блоков превышает предел высоты мира 320 блоков, а поверхность 490 млн блоков делает постройку неподъёмной. Рекомендуемый масштаб 1:100 даёт 490 × 100 блоков и 49 000 блоков поверхности — объём, доступный коллективной постройке. Район города площадью 1 км² в этом масштабе занимает 10 × 10 блоков.

## 34.4. Назначение виртуальной модели

Проектная экспертиза: проверка планировки, пешей доступности и светового климата до начала строительства.

Участие жителей: методика, отработанная ООН-Хабитат в программе Block by Block, показывает, что Minecraft снижает порог входа для людей без архитектурной подготовки и даёт проектировщикам обратную связь от будущих жителей.

Образование: школьники и студенты получают пространственное представление о станции, которое чертёж не даёт.

Цифровой двойник: модель синхронизируется с расчётными параметрами публикации, и каждое изменение планировки проверяется по балансам.

## 34.5. Проектные положения

Двадцать городов по 50 000 человек, каждый самодостаточен по жизнеобеспечению.

Виртуальный двойник в масштабе 1:100 для всей станции и 1:10 для районов детальной планировки.

Методика участия жителей — по образцу Block by Block.

Синхронизация модели с расчётными таблицами публикации.

ENGLISH EDITION

# 34. CITIES: REAL AND VIRTUAL

Item 12.5 describes urbanism as division of the habitable surface. The present section answers two questions: how the cities of the station should be arranged, and how to design them when a full-scale model is impossible.

## 34.1. Density and area

The habitable area of the two cylinders is 2,513.2 km² at a population of 1,000,000, that is a density of 398 persons/km². This is five times below the density of a typical terrestrial city and several times above rural. At a norm of 120 m² of building per person, cities occupy 120 km², or 4.8 % of the habitable surface; open space at a norm of 25 m² per person is 25 km².

Table 34.1 — Urban indicators of the station

| Quantity | Value |
| --- | --- |
| Habitable area | 2,513.2 km² |
| Density | 398 persons/km² |
| Building area | 120 km² (4.8 %) |
| Open space | 25 km² |
| Cities of 50,000 people | 20 |
| Area of one city | 6.0 km² |
| City radius | 1.38 km |

## 34.2. Real city: recommendations

Twenty cities of 50,000 people, not one metropolis: every city must be self-sufficient in food, water, energy and emergency compartmentation.

Radius 1.38 km against a cylinder radius of 5 km: the city occupies less than half the cylinder width, and walkability is preserved in all directions.

Zoning by climatic belt (item 2.4): a city in the tropical zone and a city in the cold zone are different engineering tasks and cannot be designed with one standard solution.

Orientation: the cylinder’s long axis is the only direction where the horizon is unconstrained (item 22.1). Streets and views should be oriented predominantly along the axis.

Dark band: half the cylinder length is deprived of direct OSB light; cities must be designed with artificial lighting and light climate in mind (item 22.3).

Acoustics: the shell’s natural frequencies lie below the audible range (item 7.2), and city noise finds no reflection from the walls — the layout must be open.

## 34.3. Virtual twin in Minecraft

A full-scale model of the station is impossible: a cylinder 49 km long and 10 km in diameter fits in no pavilion. A virtual twin solves this, and Minecraft proves suitable because its block geometry corresponds naturally to the orthogonal layout of the cylinder.

Table 34.2 — Virtual twin: scales

| Scale | Size, blocks | Surface | Suitability |
| --- | --- | --- | --- |
| 1:1 | 49,000 × 10,000 | 490 million | Does not fit by height |
| 1:100 | 490 × 100 | 49,000 | Recommended |
| 1:200 | 245 × 50 | 12,250 | Overview |
| 1:1000 | 49 × 10 | 490 | Too coarse |

Scale 1:1 is technically achievable in length and diameter in plan but not in height: a diameter of 10,000 blocks exceeds the world height limit of 320 blocks, and a surface of 490 million blocks makes building impracticable. The recommended scale of 1:100 gives 490 × 100 blocks and 49,000 blocks of surface — a volume accessible to collective building. A district of 1 km² occupies 10 × 10 blocks at that scale.

## 34.4. Purpose of the virtual model

Design review: checking layout, walkability and light climate before construction begins.

Resident participation: the method proven by UN-Habitat in the Block by Block programme shows that Minecraft lowers the threshold for people without architectural training and gives designers feedback from future residents.

Education: schoolchildren and students get a spatial understanding of the station that a drawing does not give.

Digital twin: the model is synchronised with the computational parameters of the publication, and every layout change is checked against the balances.

## 34.5. Design requirements

Twenty cities of 50,000 people, each self-sufficient in life support.

Virtual twin at 1:100 for the whole station and 1:10 for detailed-planning districts.

Resident participation method — on the Block by Block model.

Synchronisation of the model with the publication’s computational tables.

СПИСОК ЛИТЕРАТУРЫ РАЗДЕЛА / REFERENCES OF THIS SECTION

1. UN-Habitat. The Block by Block Playbook: Using Minecraft as a Participatory Design Tool in Urban Design and Governance. Nairobi: UN-Habitat, 2021.

1. UN-Habitat. The Block by Block Playbook: Using Minecraft as a Participatory Design Tool in Urban Design and Governance. Nairobi: UN-Habitat, 2021.

2. Benjamins R., Vieira C., Ferreira B. Minecraft as a tool for engaging children in urban planning: a case study in Tirol town, Brazil // ISPRS International Journal of Geo-Information. 2020. Vol. 9, No. 3. 170.

2. Benjamins R., Vieira C., Ferreira B. Minecraft as a tool for engaging children in urban planning: a case study in Tirol town, Brazil // ISPRS International Journal of Geo-Information. 2020. Vol. 9, No. 3. 170.